Plan B-11-1, McLellan’s Brook, 1912 (Eigg Mountain Settlement History)

…apparently using magnetic north of 1912 (23º 29’ declination)

-surveyed by Albert McKean

Begin: marked with stake and stones

1...thence N 5º E

…41.85 (327.7 yds) to ccc 1886 birch

2...thence N 89º E

…11.30 (248.6 yds) to ccc 1886 dead beech

3...thence N 5º E

…164.30 (3614.6 yds) to birch 6”

4...thence N 87º E

…78.16 (1719.52 yds) to birch 7”

5...thence N 5º E

…3.06 (67.32 yds) to stake and stone

6...thence S 84º E

…47.76 (1050.72 yds) to fir 8”

7...thence S 4º E

…122.50 (2695 yds) to stake and stones

8...thence N 48º W

…80.00 (1760 yds) to stake and stones

9...thence S 4º E

…63.00 (1386 yds) to birch 16” dia.

10...thence S 84º E

…13.00 (286 yds)

11...thence S 4º E

…31.00 (682 yds)

12…thence S 84º E

…61.00 (1342 yds)

13…thence S 4º E

…5.50 (121 yds)

14…thence S 84º E

…32.00 (704 yds)

15…thence S 4º E

…6.85 (150.7 yds) to stake and stones

16…thence N 84º W

…172.00 (3784 yds)  to place of beginning.

Plus an additional line

…beginning at birch 16” dia.

…thence S 84º E

…93.00 (2046 yds) to fallen maple rampike